HW2

  - Loading the data and plotting the sensor locations

  - Extract Oz channel data for each condition

- Compute the power spectral density (PSD) of the entire eyes open and eyes closed conditions for channel “Oz”:

- For only the eyes-closed condition, identify the peak alpha frequency (i.e., 8-12 Hz) over channel “Oz” for each subject:

peak_alpha_frequencies for subjects = [11, 10, 9, 11, 10]

Plot the topographies for the eyes open and eyes closed conditions.

- PSD for teeth clenched condition using channels “T7” and “T8:

- Plotting the teeth clenched topographies at 10, 25, 40, and 65 Hz for each subject:

- Extract Fz channel data for blinking:

- Compute the power spectral density (PSD) of the entire blinking conditions for channel “Fz” and computing the peak frequency:

- I developed an eye blink detector using a sliding window approach on EEG data. Each window was of length 256 samples (equivalent to 1 second) with an overlap of 30%, ensuring a robust and dense coverage of the entire data sequence. The data was band passed (.5-30 Hz) to get rid of the noises that might be considered as eye blinks, and also squared to improve its resilience against minor fluctuations and to accentuate the amplitude of potential blinks. A dynamic threshold for peak detection was then computed for each windowed segment based on the formula: (mean of the squared data plus 2.5 times its standard deviation). Peaks, indicative of eye blinks, were subsequently identified within these windowed segments by determining points where the amplitude exceeded the computed threshold. To maintain accuracy and avoid counting the same blink from adjacent overlapping windows, a minimum distance :(length 128 samples (equivalent to 0.5 second)) constraint was enforced between successive peaks. After processing the entire dataset, the detected peaks from all the windows were collated, and any duplicates arising from the overlap mechanism were removed. This procedure was iteratively executed for each subject's data, resulting in a list of detected blink instances. By harnessing a blend of amplitude thresholding, peak detection, and the sliding window technique with its inherent overlap, the method offers a meticulous and precise identification of eye blinks.

- To validate the performance of the developed blink detector, I randomly selected five instances from the detected blinks. Topographic maps for these instances are presented. In most of these topographic representations, I observed heightened activity in the frontal area. This prominent frontal activity serves as evidence supporting the accuracy and effectiveness of the developed detector.

# Code Section:

HW2

The data consists of 4 runs related closed eyes, open eyes, blinking, and clenching tasks:
each a 2 dimensional matrix: samples (2560) * channels (16)
Channels are as follows (ordered from 1 to 16) :
1-FPz 2-Fz 3-T7 4-T8 5-C3 6-C4 7-C5 8-C6 9-CP3 10-CP4 11-Cz 12-CPz 13-Pz 14-PO7 15-PO8 16-Oz
sampling rate: 256 Hz,
task duration: 1 min,
number of channels: 16

In [1]:

import numpy as np

from scipy.io import loadmat

import mne

import matplotlib.pyplot as plt

from scipy.signal import welch

from scipy.signal import find_peaks

from scipy.signal import butter, lfilter

In [ ]:

# 1. Load the .mat file

filenames = ["S1_data.mat", "S2_data.mat", "S3_data.mat", "S4_data.mat", "S5_data.mat"]

matrices = [loadmat(filename) for filename in filenames]

print('type(matrices):', type(matrices), 'len(matrices):', len(matrices), 'type(matrices[0]):', type(matrices[0]))

# type(matrices): <class 'list'> len(matrices): 5 type(matrices[0]): <class 'dict'>,

#So, matrices[i] refers to ith matrix

# Print the keys in the first loaded matrix

print('matrices[0].keys():', matrices[0].keys()) # matrices[0].keys(): dict_keys(['__header__', '__version__', '__globals__', 'data'])

#so matrices[i]['data'] refers to the data of the ith matrix

print(type(matrices[0]['data']), matrices[0]['data'].shape, len(matrices[0]['data'][0, 0])) #<class 'numpy.ndarray'> (1, 1) 4

#so, matrices[i]['data'][0, 0][j] refers to the jth array of the ith matrix with shape (15360, 16): EyesOpen(j=0), EyesClosed(j=1), Blink(j=2), Clench(j=3), i=0,... ,4

########################################################################################################################################################################

EyesOpen=[]

EyesClosed=[]

Blink=[]

Clench=[]

for i in range( len(matrices)):

for j in range(len(matrices[0]['data'][0, 0])):

if j==0:

EyesOpen.append(matrices[i]['data'][0, 0][j])

elif j==1:

EyesClosed.append(matrices[i]['data'][0, 0][j])

elif j==2:

Blink.append(matrices[i]['data'][0, 0][j])

else:

Clench.append(matrices[i]['data'][0, 0][j])

print('len(EyesOpen):', len(EyesOpen), 'len(EyesClosed):', len(EyesClosed), 'len(Blink):', len(Blink), 'len(Clench):', len(Clench))

#len(EyesOpen): 5 len(EyesClosed): 5 len(Blink): 5 len(Clench): 5

EyesOpen_np=np.array(EyesOpen)

EyesClosed_np=np.array(EyesClosed)

Blink_np=np.array(Blink)

Clench_np=np.array(Clench)

print('EyesOpen_np.shape:', EyesOpen_np.shape, 'EyesClosed_np.shape:', EyesClosed_np.shape, 'Blink_np.shape:', Blink_np.shape, 'Clench_np.shape:', Clench_np.shape)

#EyesOpen_np.shape: (5, 15360, 16) EyesClosed_np.shape: (5, 15360, 16) Blink_np.shape: (5, 15360, 16) Clench_np.shape: (5, 15360, 16)

######################################################################################################################################

# Read the eloc16C2.txt

with open('eloc16C2.txt', 'r') as f:

lines = [line.strip() for line in f.readlines() if line.strip()]  # This removes any empty lines

# Check that you're only processing 16 lines

if len(lines) != 16:

print(f"Warning: Expected 16 lines but found {len(lines)} lines.")

for line in lines:

print(line)  # This will print out all lines so you can inspect them

else:

# Extract channel names, theta, and radius

channel_names = [line.split()[3].replace('.', '') for line in lines]

theta = np.array([float(line.split()[1])-90 for line in lines[0:]]) * np.pi / 180.0  # Convert to radians

radius = np.array([float(line.split()[2]) for line in lines[0:]])

# Convert polar to Cartesian

x = radius * np.cos(theta)/5

y = -radius * np.sin(theta)/5

z = np.zeros_like(x)  # default z-coordinate for all channels

ch_pos = dict(zip(channel_names, zip(x, y, z)))

montage = mne.channels.make_dig_montage(ch_pos, coord_frame='head')

info = mne.create_info(ch_names=channel_names, sfreq=256, ch_types='eeg')

info.set_montage(montage)

# Plot the montage

montage.plot(show_names=True)

In [3]:

# Extract Oz channel data for each condition

eyes_open_oz = EyesOpen_np[:, :, 15]  # The 16th channel (0-based indexing)

eyes_closed_oz = EyesClosed_np[:, :, 15]

n_s = eyes_closed_oz.shape[0]

fig, axes = plt.subplots(n_s, 1, figsize=(10, 2*n_s))

for i in range(n_s):

axes[i].plot(eyes_open_oz[i], label='Eyes Open')

axes[i].plot(eyes_closed_oz[i], label='Eyes Closed')

axes[i].set_title(f'Sunject {i + 1}')

axes[i].set_xlabel('Time Points')

axes[i].set_ylabel('Amplitude')

axes[i].legend()

plt.tight_layout()

plt.show()

(a) Compute the power spectral density (PSD) of the entire eyes open and eyes closed conditions for channel “Oz”

In [4]:

# a.

# Define the sampling frequency and parameters for the Welch method

fs = 256  # Given

nperseg = fs  #256 samples for a 1-second window

noverlap = nperseg // 2  # 50% overlap

# Compute PSD for each subject

frequencies, psd_open = welch(eyes_open_oz, fs=fs, nperseg=nperseg, noverlap=noverlap, axis=1)

_, psd_closed = welch(eyes_closed_oz, fs=fs, nperseg=nperseg, noverlap=noverlap, axis=1)

# Filtering the frequencies and corresponding PSD values to be within the 0-70 Hz range

mask = (frequencies >= 0) & (frequencies <= 70)

filtered_frequencies = frequencies[mask]

filtered_psd_open = psd_open[:, mask]

filtered_psd_closed = psd_closed[:, mask]

psd_open_log=10*np.log10(filtered_psd_open)

psd_closed_log=10*np.log10(filtered_psd_closed)

fig, axes = plt.subplots(n_s, 1, figsize=(10, 2*n_s))

for i in range(n_s):

axes[i].plot(psd_open_log[i], label='Eyes Open')

axes[i].plot(psd_closed_log[i], label='Eyes Closed')

axes[i].set_title(f'Subject {i + 1} - Power Spectral Density (PSD) for Channel "Oz"')

axes[i].set_xlabel('Frequency (Hz)')

axes[i].set_ylabel('PSD (dB/Hz)')

axes[i].legend()

axes[i].grid(True, which='both')

plt.tight_layout()

plt.show()

In [5]:

# b.

alpha_mask = (filtered_frequencies >= 8) & (filtered_frequencies <= 12)

alpha_psd = psd_closed_log[:, alpha_mask]

peak_alpha_indices = np.argmax(alpha_psd, axis=1)

peak_alpha_frequencies = filtered_frequencies[alpha_mask][peak_alpha_indices]

#peak_alpha_frequencies = array([11., 10.,  9., 11., 10.])

num_subjects, num_timepoints, num_channels = EyesOpen_np.shape # (5, 15360, 16)

# Preallocate arrays for storing the PSD data

psd_open_all = np.empty((num_subjects, num_channels, len(frequencies)))

psd_closed_all = np.empty((num_subjects, num_channels, len(frequencies)))

# Calculate the PSD for all channels and subjects

for channel in range(num_channels):

_, psd_open_all[:, channel, :] = welch(EyesOpen_np[:, :, channel], fs=fs, nperseg=nperseg, noverlap=noverlap, axis=1) #shape: (5, 16, 129)

_, psd_closed_all[:, channel, :] = welch(EyesClosed_np[:, :, channel], fs=fs, nperseg=nperseg, noverlap=noverlap, axis=1) #shape: (5, 16, 129)

open=[]

closed=[]

for subject in range(num_subjects):

# Extract the peak alpha frequency for this subject

peak_freq = peak_alpha_frequencies[subject]

# Extract PSD values at the peak alpha frequency for both eyes-open and eyes-closed conditions

closed_peak_psd_values = psd_closed_all[subject, :, frequencies == peak_freq].squeeze()

open_peak_psd_values = psd_open_all[subject, :, frequencies == peak_freq].squeeze()

closed.append(closed_peak_psd_values)

open.append(open_peak_psd_values)

# Determine consistent color limits across conditions for each sunject

global_vmin = min(np.min(closed_peak_psd_values), np.min(open_peak_psd_values))

global_vmax = max(np.max(closed_peak_psd_values), np.max(open_peak_psd_values))

fig, axes = plt.subplots(1, 2, figsize=(6,2.5))

# Eyes-closed condition

im1, _ = mne.viz.plot_topomap(closed_peak_psd_values, info, cmap='jet', axes=axes[0], show=False)

im1.set_clim(global_vmin, global_vmax)

axes[0].set_title(f'Subject {subject + 1} - Eyes Closed\nPeak Alpha at {peak_freq:.2f} Hz')

plt.colorbar(im1, ax=axes[0])

# Eyes-open condition

im2, _ = mne.viz.plot_topomap(open_peak_psd_values, info, cmap='jet', axes=axes[1], show=False)

im2.set_clim(global_vmin, global_vmax)

axes[1].set_title(f'Subject {subject + 1} - Eyes Open\nPeak Alpha at {peak_freq:.2f} Hz')

plt.colorbar(im2, ax=axes[1])

plt.tight_layout()

plt.show()

In [6]:

# c.

# Extract channels “T7” and “T8 (2 and 3) data for each condition

Clench_np_T7 = Clench_np[:, :, 2]

Clench_np_T8 = Clench_np[:, :, 3]

fs = 256

nperseg = fs  #256 samples for a 1-second window

noverlap = nperseg // 2  # 50% overlap

# Compute PSD for each subject

frequencies, psd_Clench_T7 = welch(Clench_np_T7, fs=fs, nperseg=nperseg, noverlap=noverlap, axis=1)

_, psd_Clench_T8 = welch(Clench_np_T8, fs=fs, nperseg=nperseg, noverlap=noverlap, axis=1)

# Filtering the frequencies and corresponding PSD values to be within the 0-70 Hz range

mask = (frequencies >= 0) & (frequencies <= 70)

filtered_frequencies = frequencies[mask]

filtered_psd_Clench_T7 = psd_Clench_T7[:, mask]

filtered_psd_Clench_T8 = psd_Clench_T8[:, mask]

psd_Clench_T7_log=10*np.log10(filtered_psd_Clench_T7)

psd_Clench_T8_log=10*np.log10(filtered_psd_Clench_T8)

fig, axes = plt.subplots(n_s, 1, figsize=(10, 2*n_s))

for i in range(n_s):

axes[i].plot(psd_Clench_T7_log[i], label='psd_Clench_T7')

axes[i].plot(psd_Clench_T8_log[i], label='psd_Clench_T8')

axes[i].set_title(f'Subject {i + 1} - Power Spectral Density (PSD) for Clench task')

axes[i].set_xlabel('Frequency (Hz)')

axes[i].set_ylabel('PSD (dB/Hz)')

axes[i].legend()

axes[i].grid(True, which='both')

plt.tight_layout()

plt.show()

In [7]:

#d.

# Preallocate arrays for storing the PSD data

PSD_Clench = np.empty((num_subjects, num_channels, len(frequencies)))

# psd_Clench_T8 = np.empty((num_subjects, num_channels, len(frequencies)))

clench_freq=[10, 25, 40, 65]

# Calculate the PSD for all channels and subjects

for channel in range(16):

_, PSD_Clench[:, channel, :] = welch(Clench_np[:, :, channel], fs=fs, nperseg=nperseg, noverlap=noverlap, axis=1) #shape: (5, 16, 129)

fig, axes = plt.subplots(5, len(clench_freq), figsize=(3 * len(clench_freq), 1.5 * 5))

for subject in range(5):

# Using a list comprehension to gather the PSD values for each frequency in clench_freq

psd_values_for_freqs = [PSD_Clench[subject, :, frequencies == freq].squeeze() for freq in clench_freq]

# Get global vmin and vmax for colorbar scaling for the current subject across all frequencies

global_vmin = np.min(psd_values_for_freqs)

global_vmax = np.max(psd_values_for_freqs)

for idx, frequency in enumerate(clench_freq):

clenched_psd_values = psd_values_for_freqs[idx]

im, _ = mne.viz.plot_topomap(clenched_psd_values, info, cmap='jet', axes=axes[subject, idx], show=False)

im.set_clim(global_vmin, global_vmax)

axes[subject, idx].set_title(f'Subject {subject + 1} - Frequency {frequency} Hz')

# Add individual colorbar for each subject at the rightmost column of the subplot array

fig.colorbar(im, ax=axes[subject, -1], orientation='vertical', pad=0.1)

plt.tight_layout()

plt.show()

In [8]:

# 2.

# Extract Fz channel data for blinking

eyes_blink_Fz = Blink_np[:, :, 1]  # The 16th channel (0-based indexing)

n_s = eyes_blink_Fz.shape[0]

fig, axes = plt.subplots(n_s, 1, figsize=(10, 2*n_s))

for i in range(n_s):

axes[i].plot(eyes_blink_Fz[i], label='Eyes Open')

axes[i].set_title(f'Sunject {i + 1}')

axes[i].set_xlabel('Time Points')

axes[i].set_ylabel('Amplitude')

axes[i].legend()

plt.tight_layout()

plt.show()

In [9]:

# 2.

# Define the sampling frequency and parameters for the Welch method

fs = 256  # Given

nperseg = fs  #256 samples for a 1-second window

noverlap = nperseg // 2  # 50% overlap

# Compute PSD for each subject

frequencies, psd_blink_Fz = welch(eyes_blink_Fz, fs=fs, nperseg=nperseg, noverlap=noverlap, axis=1)

# Filtering the frequencies and corresponding PSD values to be within the 0-50 Hz range,

mask = (frequencies >= 0) & (frequencies <= 50)

filtered_frequencies = frequencies[mask]

filtered_psd_blink_FZ = psd_blink_Fz[:, mask]

psd_blink_log_FZ=10*np.log10(filtered_psd_blink_FZ)

# calculate the peak frequency

blink_mask = (filtered_frequencies >= 1) & (filtered_frequencies <= 15)

blink_psd = psd_blink_log_FZ[:, blink_mask]

peak_blink_indices = np.argmax(blink_psd, axis=1)

peak_blink_frequencies = filtered_frequencies[blink_mask][peak_blink_indices]

print('peak_blink_frequencies:', peak_blink_frequencies)

# peak_blink_frequencies: [3. 2. 3. 3. 2.]

# Now modify your plotting loop:

fig, axes = plt.subplots(n_s, 1, figsize=(10, 2*n_s))

for i in range(n_s):

axes[i].plot(filtered_frequencies, psd_blink_log_FZ[i], label='FZ')

# Add a red dotted line for the peak frequency

peak_freq = peak_blink_frequencies[i]

peak_power = psd_blink_log_FZ[i][filtered_frequencies == peak_freq]

axes[i].axvline(x=peak_freq, color='red', linestyle='--')

# Annotate the point with frequency and power value

annotation_text = f'({peak_freq:.2f} Hz, {peak_power[0]:.2f} dB/Hz)'

axes[i].annotate(annotation_text, xy=(peak_freq, peak_power[0]), xycoords='data',

xytext=(-100, 10), textcoords='offset points',

arrowprops=dict(arrowstyle="->"))

axes[i].set_title(f'Subject {i + 1} - Power Spectral Density (PSD) for Channel "Fz"')

axes[i].set_xlabel('Frequency (Hz)')

axes[i].set_ylabel('PSD (dB/Hz)')

axes[i].legend()

axes[i].grid(True, which='both')

plt.tight_layout()

plt.show()

peak_blink_frequencies: [3. 2. 3. 3. 2.]

The eye blink detector was developed using a sliding window approach on EEG data. Each window was of length 256 samples (equivalent to 1 second) with an overlap of 30%, ensuring a robust and dense coverage of the entire data sequence. Within each window, the data was band passed (.5-30 Hz) to get rid of the noises that might be considered as eye blinks, and also squared to improve its resilience against minor fluctuations and to accentuate the amplitude of potential blinks. A dynamic threshold for peak detection was then computed for each windowed segment based on the formula: (mean of the squared data plus 2.5 times its standard deviation). Peaks, indicative of eye blinks, were subsequently identified within these windowed segments by determining points where the amplitude exceeded the computed threshold. To maintain accuracy and avoid counting the same blink from adjacent overlapping windows, a minimum distance :(length 128 samples (equivalent to 0.5 second)) constraint was enforced between successive peaks. After processing the entire dataset, the detected peaks from all the windows were collated, and any duplicates arising from the overlap mechanism were prudently removed. This procedure was iteratively executed for each subject's data, resulting in a list of detected blink instances. By harnessing a blend of amplitude thresholding, peak detection, and the sliding window technique with its inherent overlap, the method offers a meticulous and precise identification of eye blinks.

In [16]:

def compute_threshold(eeg_data, k=2.5):

"""

Computes the amplitude threshold for blink detection.

Parameters:

- eeg_data: 1D array representing EEG data.

- k: constant multiplier for the standard deviation.

Returns:

- Threshold for blink detection.

"""

return np.mean(eeg_data) + k * np.std(eeg_data), #std: Compute the standard deviation

def detect_blinks(eeg_data, threshold):

"""

Detects eye blinks based on amplitude thresholding.

Parameters:

- eeg_data: 1D array representing EEG data.

- threshold: amplitude threshold for detecting blinks.

Returns:

- blink_times: indices where blinks are detected.

"""

blink_times = np.where(np.abs(eeg_data) > threshold)[0]

return blink_times

def butter_bandpass(lowcut, highcut, fs, order=5):

nyq = 0.5 * fs

low = lowcut / nyq

high = highcut / nyq

b, a = butter(order, [low, high], btype='band')

return b, a

def butter_bandpass_filter(data, lowcut, highcut, fs, order=5):

b, a = butter_bandpass(lowcut, highcut, fs, order=order)

y = lfilter(b, a, data)

return y

# Apply filter

lowcut = 0.5

highcut = 30.0

filtered_eyes_blink_Fz = butter_bandpass_filter(eyes_blink_Fz, lowcut, highcut, fs)

In [17]:

distance_between_peaks = 128  # Half of one second between peaks; adjust as needed

window_size = 256 # 1 seconds window size

window_step = int(window_size * 0.7)  # 30% overlap

blink_times_all_subjects = []  # Master list to store blink times for all subjects

# Process each epoch of the data

for subject_num, epoch_data in enumerate(filtered_eyes_blink_Fz, start=1):

squared_data = epoch_data**2  # Square the data for more resilience

threshold = compute_threshold(squared_data)  # Compute threshold for the squared data

all_peaks = []

# Slide the window through the data

for start in range(0, len(squared_data) - window_size + 1, window_step):

end = start + window_size

windowed_data = squared_data[start:end]

# Detect peaks in the windowed data

peaks, _ = find_peaks(windowed_data, height=threshold, distance=distance_between_peaks)

peaks = peaks + start  # Adjust peak indices for the entire data

all_peaks.extend(peaks)

# Remove duplicate peaks

all_peaks = list(set(all_peaks))

all_peaks.sort()

# Append the blink times for this epoch to the master list

blink_times_all_subjects.append(all_peaks)

# Print the number of detected blinks

# print(f"Detected {len(all_peaks)} blinks in this epoch.")

# Visualize detected blinks

plt.figure(figsize=(15, 3))

plt.plot(squared_data)

plt.plot(all_peaks, squared_data[all_peaks], "ro")

# Set title to indicate the subject number

plt.title(f"Detected Blinks for Subject {subject_num}")

# Annotate the number of peaks detected

plt.annotate(f"Detected Blinks for Subject_{subject_num}: {len(all_peaks)}",

xy=(0.95, 0.95), xycoords='axes fraction',

fontsize=10,

xytext=(-5, -5), textcoords='offset points',

ha='right', va='top')

plt.show()

# Convert the master list to an array for convenience

blink_times_array = np.array(blink_times_all_subjects)

print(blink_times_array)

[list([1940, 2450, 2933, 3046, 3132, 3476, 3668, 4000, 4198, 4357, 4569, 5171, 5498, 5753, 6268, 6420, 6803, 6947, 7448, 8234, 8893, 9175, 9360, 9571, 9980, 10145, 10283, 10513, 10682, 10814, 10847, 11036, 11206, 11370, 11472, 11709, 11849, 11999, 12754, 12959, 13452, 13591, 13770, 13901, 14141, 14325, 14511, 15110])

list([67, 224, 273, 643, 926, 1071, 1510, 1713, 1831, 2034, 2880, 3091, 3134, 3688, 4429, 4559, 4846, 5621, 6035, 6404, 6787, 6983, 8209, 8386, 8530, 8667, 8804, 9355, 9963, 10109, 10258, 10338, 11382, 11778, 12155, 12177, 12582, 12825, 13248, 13708, 13899, 14226, 14492, 14749, 14791, 14900])

list([280, 496, 731, 959, 995, 1190, 1423, 1660, 1940, 2182, 2384, 2605, 2861, 2897, 3091, 3348, 3551, 3585, 3822, 4001, 4191, 4393, 4643, 4757, 4998, 5164, 5359, 5555, 6372, 6771, 7585, 8039, 8251, 8485, 8658, 8900, 9078, 9314, 9522, 9717, 11298, 12038, 12320, 12434, 12684, 12731, 14620, 14842, 15061, 15270])

list([228, 511, 1148, 1404, 1529, 1744, 2002, 2196, 2370, 2531, 2761, 2934, 2952, 3199, 3346, 3535, 3772, 3966, 4181, 4396, 4630, 4789, 5023, 5410, 5602, 5822, 6011, 6395, 6591, 6799, 7309, 7633, 8646, 8788, 8968, 9344, 9728, 10080, 10329, 10448, 10664, 10930, 11023, 11154, 11284, 11386, 11564, 11758, 12050, 12189, 12259, 12537, 12653, 13047, 13529, 13774, 13962, 14255, 14628, 14796, 14992])

list([8, 2459, 2515, 2868, 3231, 3626, 3941, 4298, 4676, 5048, 5411, 5770, 6096, 6351, 6592, 7298, 7527, 7759, 7989, 8170, 8432, 8694, 8957, 9241, 9382, 9390, 9503, 9788, 10121, 10804, 11078, 11352, 11607, 11914, 12115, 12310, 12604, 12816, 13059, 13314, 13357, 14113, 14166, 14456, 14712])]

blink_times_array = np.array(blink_times_all_subjects)

To validate the performance of the developed blink detector, we randomly selected five instances from the detected blinks. Topographic maps for these instances are presented. In most of these topographic representations, we observed heightened activity in the frontal area. This prominent frontal activity serves as evidence supporting the accuracy and effectiveness of our developed detector.

In [19]:

# Validation

# Reshape the data to (n_epochs, n_channels, n_times)

Blink = np.transpose(Blink_np, (0, 2, 1))

# Create an EpochsArray object using your provided info

events = np.array([[i, 0, 1] for i in range(Blink.shape[0])])

epochs = mne.EpochsArray(Blink, info, events=events)

# Precompute the global minimum and maximum across subjects and blink times for consistent colormap scaling

global_vmin = np.inf

global_vmax = -np.inf

for i in range(Blink.shape[0]):

random_blink_times = np.random.choice(blink_times_array[i], 5, replace=False)

for blink_time in random_blink_times:

time_point = blink_time / epochs.info['sfreq']

data = epochs[i].average().data[:, epochs.time_as_index([time_point])[0]]

global_vmin = min(global_vmin, data.min())

global_vmax = max(global_vmax, data.max())

fig, axs = plt.subplots(Blink.shape[0], 5, figsize=(15, 10))

for i in range(Blink.shape[0]):

# Randomly select 5 blink times for this subject

random_blink_times = np.random.choice(blink_times_array[i], 5, replace=False)

# Convert sample points to time in seconds

random_time_points = random_blink_times / epochs.info['sfreq']

# Get data values for the randomly selected times and determine their min and max

subject_data = epochs[i].average().data # This Evoked object contains the grand average (averaged over trials or epochs) of the data for each channel.

subject_values_at_times = [subject_data[:, int(tp * epochs.info['sfreq'])] for tp in random_time_points]

subject_vmin = min([np.min(val) for val in subject_values_at_times])

subject_vmax = max([np.max(val) for val in subject_values_at_times])

# For each of these blink times

for j, time_point in enumerate(random_time_points):

# Plot the topomap for this time point

data_at_time = subject_data[:, int(time_point * epochs.info['sfreq'])]

im1, _ = mne.viz.plot_topomap(data_at_time, epochs.info, cmap='jet', axes=axs[i, j], show=False)

im1.set_clim(subject_vmin, subject_vmax)

axs[i, j].set_title(f"Subject {i+1}, Time: {time_point:.2f}s")

fig.colorbar(im1, ax=axs[i, 4], orientation='vertical', pad=0.05)

plt.tight_layout()

plt.show()